GRIS – Detachment Inspection Checklist

| Project: |  |
| --- | --- |
| Unit: |  |
| Remarks: |  |

| Position | Item | Potential Detachment Concern | Residual risk | Expected Qty | Pre-Test Qty Verified | Pre-Test Associated Control / Verification Point |  | Pre-Test Remarks | Post-Test Qty Verified |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Cover retaining screw | Backing out | Low | 8 |  | Torque mark aligned | ☐ |  |  |
|  |  |  |  |  |  | Retention tape present and intact | ☐ |  |  |
|  |  |  |  |  |  | No visible loosening or backing out | ☐ |  |  |
| 2 | Module joining shoulder screw | Backing out | Low | 8 |  | Torque level verified | ☐ |  |  |
| 3 | Extender retaining screw | Backing out | Very Low | 6 |  | Torque level verified | ☐ |  |  |
| 4 | ELCID magnetic sensor fastening screw | Backing out / Pull-out | Very Low | 2 |  | Torque level verified | ☐ |  |  |
| 5 | Motor cover screw | Backing out / Pull-out | Low | 8 |  | Torque mark aligned | ☐ |  |  |
|  |  |  |  |  |  | Retention tape present and intact | ☐ |  |  |
| 6 | ELCID magnetic sensor | Fracture | Very Low | 2 |  | No visible damage or degradation | ☐ |  |  |
| 7 | Camera retaining screw | Backing out / Pull-out | Low | 10 |  | Torque mark aligned | ☐ |  |  |
| 8 | Gear cover | Fracture / Disengagement | Very Low | 2 |  | Fit fully seated | ☐ |  |  |
|  |  |  |  |  |  | Retention tape present and intact | ☐ |  |  |
| 9 | Track belt | Derailment | Very Low | 2 |  |  |  |  |  |
| 10 | Recovery hook | Backing out / Pull-out | Very Low | 2 |  | Torque mark aligned | ☐ |  |  |
| 11 | Wheel axle shoulder screw | Backing out | Low | 6 |  | Torque mark aligned | ☐ |  |  |
| 12 | Main connector housing screw | Backing out | Very Low | 2 |  | Torque mark aligned | ☐ |  |  |
|  |  |  |  |  |  | Retention tape present and intact | ☐ |  |  |
| 13 | Main connector latch screw | Backing out | Very Low | 2 |  | Mechanical retention engaged | ☐ |  |  |
| 14 | Bottom camera position locking screw | Backing out | Very Low | 2 |  | Mechanical retention engaged | ☐ |  |  |
| 15 | Central slot guide | Disengagement | Very Low | 2 |  | Fit fully seated | ☐ |  |  |
|  |  |  |  |  |  | No movement or disengagement | ☐ |  |  |
| 16 | Leaf spring retaining screw | Backing out | Low | 2 |  | Torque mark aligned | ☐ |  |  |
|  |  |  |  |  |  | No visible loosening or backing out | ☐ |  |  |
| 17 | Leaf spring | Fracture | Very Low | 1 |  | No cracks or permanent deformation | ☐ |  |  |
| 18 | Impactor grub screw | Backing out | Very Low | 2 |  | Retention tape present and intact | ☐ |  |  |
|  |  |  |  |  |  | Light torque confirmation | ☐ |  |  |
| 19 | Track wheels | Fracture | Very Low | 6 |  | No visible damage or degradation | ☐ |  |  |
| 20 | Magnets retaining screw | Backing out | Low | 8 |  | Torque mark aligned | ☐ |  |  |
| 21 | Impact solenoid | Detachment | Very Low | 1 |  | Fully seated in mount | ☐ |  |  |
| 22 | Camera lens | Backing out | Very Low | 6 |  | No movement under light torque | ☐ |  |  |

| Pre-Test Verification |  |
| --- | --- |
| Verified by: |  |
| Signature: |  |
| Date: |  |

| Post-Test Verification |  |
| --- | --- |
| Verified by: |  |
| Signature: |  |
| Date: |  |

Position Identification

Figure 1. Position Identification - GRIS isometric top view

Figure 2. Position Identification - GRIS isometric bottom view